// LED Pins

const int redLED = 8;

const int yellowLED = 9;

const int greenLED = 10;

// Push Button Pin

const int buttonPin = 2;

unsigned long startTime;

unsigned long reactionTime;

void setup() {

pinMode(redLED, OUTPUT);

pinMode(yellowLED, OUTPUT);

pinMode(greenLED, OUTPUT);

pinMode(buttonPin, INPUT_PULLUP);

Serial.begin(9600);

// Random seed

randomSeed(analogRead(A0));

Serial.println("Reaction Time Game");

Serial.println("Wait for the GREEN LED and press the button!");

}

void loop() {

// Turn LEDs off

digitalWrite(redLED, LOW);

digitalWrite(yellowLED, LOW);

digitalWrite(greenLED, LOW);

Serial.println("\nNew Round Starting...");

delay(2000);

// Random LED sequence

for (int i = 0; i < 10; i++) {

int ledChoice = random(0, 3);

int randomDelay = random(200, 1001);

switch (ledChoice) {

case 0:

digitalWrite(redLED, HIGH);

delay(randomDelay);

digitalWrite(redLED, LOW);

break;

case 1:

digitalWrite(yellowLED, HIGH);

delay(randomDelay);

digitalWrite(yellowLED, LOW);

break;

case 2:

digitalWrite(greenLED, HIGH);

delay(randomDelay);

digitalWrite(greenLED, LOW);

break;

}

delay(random(100, 500));

}

// Reaction signal

Serial.println("GO!");

digitalWrite(greenLED, HIGH);

startTime = millis();

// Wait for button press

while (digitalRead(buttonPin) == HIGH) {

// Waiting

}

reactionTime = millis() - startTime;

digitalWrite(greenLED, LOW);

Serial.print("Reaction Time: ");

Serial.print(reactionTime);

Serial.println(" ms");

delay(3000);

}